Product Specifications – Harvesting Chassis Kit (Mobile Chassis + Lifting Platform)

| Design Dimensions | 2330×1925×660 mm | External Power Supply | BAT 48V/30A, 24V/30A |
| --- | --- | --- | --- |
| Gross Vehicle Weight | 300 kg (chassis) + 353 kg (lifting platform) | Braking Method | Motor braking |
| Structure | Four-wheel drive, four-wheel steering | Parking Method | Motor parking + electromagnetic brake (power-off parking) |
| Suspension | No suspension | Vertical Load (level ground) | 300 kg |
| Ground Clearance | 114 mm | Operating Speed | 7.2 km/h |
| Wheelbase | 1750 mm | Wading Depth | 130 mm |
| Track Width | 1570 mm | Range | 40 km (no load) / 30 km (full load) |
| Tire Diameter | 418 mm | Max. Climbing Angle | 30° (no load) / 10° (full load) |
| Drive Motors | 1.2 kW × 4 | Trench Crossing Width | 300 mm (no load) / 200 mm (full load) |
| Battery | 48V / 82.5Ah lithium iron phosphate (LiFePO4) battery | Obstacle Clearance Height | 110 mm (no load) / 50 mm (full load) |
| Charging Time | 4–6 h | Protection Rating | IP44 |
| Charging Method | 54.6V/20A manual charging plug | Communication | CAN 2.0B |
| Lifting Height | 1800 mm | Platform Deck Dimensions | 1500 mm × 1150 mm |